SECTION 01 5000

TEMPORARY FACILITIES AND CONTROLS

*****  The Design Professional shall include the following requirements as noted and complete this section with information for the specific project in coordination with PSFA Section 00 7200 - General Conditions*****

PART 1 - GENERAL

1.1	SUMMARY

A.	Section includes:

1.	Site mobilization plan.

2.	Temporary services:  Electrical, lighting, heating, ventilating, water, telephone, and facsimile.

3.	Fencing, barriers, and other temporary controls.

4.	Temporary erosion and sediment controls including NPDES-SWPPP requirements.

5.	Construction facilities:  Temporary buildings, sanitary facilities, access, and  parking.

6.	Protection of Work and existing facilities.

7.	Project sign.

8.	Bulletin board.

B.	Related documents and sections:

1.	Document 00 7200 - General Conditions:

a.	Paragraph  3.13:  Contractor's use of site..

b.	Paragraph  3.15:  Contractor's responsibility for cleaning.

c.	Article  10:  Safety precautions and programs.

2.	Section 01 3100: Project Management and Coordination

2.	Section 01 7000 - Execution Requirements:  Progress cleaning.

1.2	REFERENCES

A.	NFPA 10 - Standard for Portable Fire Extinguishers.

B.	NFPA 241 - Safeguarding Building Construction, Alterations, and Demolition Operations.

1.3	SITE MOBILIZATION PLAN

A.	Coordinate locations for temporary facilities with Design Professional and Owner.

B.	Based upon information indicated on Drawings, prepare site mobilization plan in accordance with requirements for site logistics plan in Subparagraph 3.13.14 in Document 00 7200 General Conditions.

C.	Present 3 copies of plan at Pre-Construction Conference in accordance with Section 01 3100 - Project Management and Coordination.

D.	Prior to mobilization, revise and resubmit to Design Professional site mobilization plan incorporating final revisions made at Pre-Construction Conference and approved by Design Professional and Owner.

1.4	TEMPORARY ELECTRICITY

*****  Include the following paragraph if Contractor is to provide and pay for temporary electricity.  *****

A.	Provide and pay for temporary electricity used during construction.  Provide service disconnect and overcurrent protection.  Provide temporary feeder as required.

*****  Include and edit the following paragraph if there is existing power at site and Owner is to pay for temporary electricity.  *****

B.	Connect to existing power source at site.  [Do not disrupt Owner's need for continuous service.]  Provide service disconnect and overcurrent protection.  Provide temporary feeder as required.  [Owner will pay cost of electricity used.  Exercise measures to conserve power.]  [Provide separate metering and reimburse Owner for cost of energy used.]

C.	Provide power outlets for construction operations with branch wiring, distribution boxes, and flexible power cords as required.

D.	Permanent convenience receptacles may be utilized during construction.

1.5	TEMPORARY LIGHTING

A.	Provide lighting for construction operations in accordance with Paragraph 3.13 in the General Conditions.  Lighting levels shall be appropriate for type and difficulty of work.  Use these minimums as guidelines:

B.	After dark, provide security lighting for interior and exterior work and storage areas.

C.	Provide branch wiring from power source to distribution boxes with lighting conductors, pigtails, and lamps as required.

D.	Maintain lighting and provide routine repairs.

E.	Permanent building lighting may be utilized during construction.  Document existing lighting system conditions at start of Work and submit report to Design Professional for approval before Work begins.  Re-lamp, replace, or repair existing fixtures at end of job to return lighting to conditions documented prior to commencement of Work.

1.6	TEMPORARY HEATING AND VENTILATING

A.	Ventilate enclosed areas to assist cure of materials, to dissipate humidity, and to prevent accumulation of dust, fumes, vapors, and gases.

B.	Provide temporary fan units to maintain clean air for construction operations.

C.	Maintain minimum ambient temperature of 50 degrees F in interior areas where construction is in progress.

***** Include the following paragraphs if project is renovation of existing building and Contractor is allowed to use existing HVAC system.  *****

D.	Use Owner's existing HVAC system to maintain specified conditions.  Owner will pay cost of energy used.  Exercise measures to conserve energy.

E.	If Owner’s existing HVAC system is temporarily insufficient or inoperable due to the Work, provide and pay for supplemental heating devices needed to maintain specified conditions and in such a manner as to prevent damage to existing building and systems.

F.	Change all HVAC filters in existing system serving area of Work at end of Project.

1.7	TEMPORARY WATER SERVICE

A.	Provide, maintain, and pay for suitable quality water service required for construction operations.

*****  Include the following paragraph if there is an existing water source on site.  Edit to reflect payment by Owner or Contractor.  *****

[A.	Connect to existing water source at site for construction operations.  Provide separate metering and reimburse Owner for cost of water used.]

[B.	Assume responsibility for temporary connections and water lines.  Upon completion, remove temporary facilities.]

1.8	COMMUNICATIONS

A.	Provide, maintain, and pay for telephone service to field office.  School telephones will not be available to Contractor’s workforce unless for an emergency.

B.	Provide, maintain, and pay for facsimile service to field office.

1.9	FENCING

A.	Provide temporary fencing around new building and materials storage site.  Completely separate construction from existing facilities, student pathways a nd related exterior areas.

B.	Type:  Panelized 6 foot high commercial grade chain link fence.  Equip with vehicular and pedestrian gates with locks.

1.10	BARRIERS AND PROTECTION

*****  Edit this article to reflect scope of project, if there are existing buildings and landscaping, and type of barriers and protection specifically required.  *****

A.	Security:  Provide to protect Work [and existing facilities] from unauthorized entry, vandalism, and theft.  [Coordinate with Owner's security program and personnel.]

B.	Barriers:  Provide to prevent unauthorized entry to construction areas and to protect existing facilities and adjacent properties from construction operations.

C.	Barricades and covered walkways:  As required by Design Professional, Owner and governing authorities for safe public access to existing buildings.

D.	Enclosures:  Provide temporary, insulated, weather tight closures of exterior openings to provide acceptable working conditions, protect Work, and prevent unauthorized entry.  Fit with lockable doors.

E.	Temporary partitions:  Provide to separate work areas from [existing building at point of connection.] [completed Work.]  Prevent penetration of dust and moisture into [existing] [completed portions of] building.

F.	Emergency exits shall be maintained during construction.  Provide separate barriers as appropriate.

G.	Protect existing detection devices such as smoke detectors and sensors from construction dust.

H.	Protect existing trees and plants designated to remain.  Replace damaged plant material.

I.	Hand-water existing trees, plants [and grass] as necessary to maintain them viable in the event that existing irrigation system is made temporarily inoperable due to the Work.  Replace dead plant material as required in the event of failure to comply with this provision.

1.11	PROTECTION OF EXISTING AND INSTALLED WORK

A.	Protect installed Work.  Control activity in immediate work area.

B.	Provide temporary and removable protection for installed products.

C.	Protect finished floors, stairs, and other surfaces from traffic, dirt, wear, and movement of heavy objects with durable sheet materials.

D.	Prohibit traffic and storage on roof surfaces and landscaped areas.

1.12	TEMPORARY FIRE PROTECTION

A.	Install and maintain temporary fire protection components.  Establish and follow procedures to protect against fire losses.  Comply with NFPA 241.

B.	Fire extinguishers:  Provide hand carried, portable, UL rated fire extinguishers of type and size recommended by NFPA 10 for building exposure conditions.  Place in accessible, convenient locations in clear view with a minimum of one extinguisher per floor.

C.	Access:  Maintain unobstructed access to fire hydrants, water supply, fire extinguishers, stairways, and access routes for fighting fires.

D.	Heating devices:  Exercise care and monitor use of temporary heaters to minimize fire risk.

E.	Store combustible materials in fire-safe containers.

F.	Volatile products:  Do not store paints, varnishes, paint removers, solvents, adhesives, cleaning rags, and other volatile products in building.  Take precautionary measures to prevent fire hazards and spontaneous combustion.

G.	Cutting and welding:  Approve in advance use of open flame cutting, welding, and soldering equipment.  Ensure that safe conditions exist before granting approval.

*****	Include the following paragraph when Project involves construction activities which may cause soils disturbance and erosion on site	*****

  - TEMPORARY EROSION AND SEDIMENT CONTROLS

- Prevent temporary collection of sediment on sidewalks, parking lots, streets and driveways.  Clean such surfaces promptly if such conditions exist due to the Work.

- National Pollution Discharge Elimination System (NPDES) permit and procedures for preparing a Storm Water Pollution Prevention Plan (SWPPP).

1.	Contractor shall determine whether Project requires an EPA NPDES storm water discharge permit in conformance with all regulations governing the disturbance of construction site areas.

2.	If storm water discharge permit is required, then both Contractor and Owner shall be designated as separate permittees and the Contractor shall do the following:

a.	Prepare a Storm Water Pollution Prevention Plan (SWPPP) document as necessary to ensure compliance with any and all NPDES construction storm water permitting plan requirements.

b.	Prepare and submit all EPA documentation and forms required of Contractor for permit.

c.	Assist Owner with preparation and submittal of all EPA documentation and forms specifically required of Owner for permit.  Provide all required project-related information to Owner as necessary.

- At Final Completion of Project, Contractor shall complete and submit documentation to EPA as required and to Design Professional as part of Project Closeout documentation package.  See Section 01 78 00 of Specifications.

- If a storm water discharge permit is not required, then the Contractor shall submit to the Design Professional and Owner prior to mobilization a signed statement containing specific written justification why such permit is not required on the Project.

- The Contractor shall manage the discharge of storm water from the site in accordance with NPDES permit and the provisions of the SWPPP.  The Contractor shall be responsible for installing and maintaining any necessary storm water control measures in accordance with control device manufacturer’s recommendations and the provisions of the SWPPP.  The Contractor shall monitor the suitability of the designated control measures and management practices to achieve the storm water quality provisions of the NPDES permit, and shall make any necessary changes to the controls and practices in order to meet the permit requirements.  The Contractor shall be responsible for updating the SWPPP and maintaining all records related to the SWPPP.  A copy of the approved SWPPP shall be kept on the jobsite at all times.  Contractor shall be liable for all fines and construction delays resulting from any governmental agency enforcement action due to failure by the Contractor to satisfy the above requirements.

- Contractor is responsible for payment of all applicable fees and permits related to SWPPP approval process and for full cost of control measures for the Project.

1.14	ACCESS

A.	Refer to Drawings for location of acceptable access routes and site entrances.  Protect existing curbs and walks traversed by construction vehicles from damage.

B.	Identify access to Contractor's work and office area with appropriate signs so that delivery personnel and others may contact Contractor.  School office shall not be used as destination for Contractor’s deliveries.

C.	Prevent unauthorized personnel from accessing school building or site through Contractor's work area.

1.15	FIELD FACILITIES

A.	[Provide and maintain a weathertight, fully equipped field office.] [Provide work station for use of Design Professional during field inspections.]

B.	Provide space for project meetings with table and chairs to accommodate 6 persons.

C.	Provide and maintain storage sheds and other facilities as required.

D.	Arrange for parking for work force in manner approved by Owner.  [Do not limit Owner's requirements for parking.]

1.16	TEMPORARY SANITARY FACILITIES

A.	Provide and maintain required sanitary facilities for work force.

B.	New  [and existing]  toilet facilities shall not be used by work force.

1.17	DRINKING WATER

A.	Provide independent source of drinking water for workforce.  School drinking fountains shall not be routinely available for Contractor’s use.

1.18	PROJECT SIGNS

*****	Include the following paragraph if Project includes PSOC funding.	*****

A.	PSFA Construction Sign.  See  Section 01 5001 .

1.	Furnish project sign and erect on site at location designated by Design Professional.

2.	Construction: 4 by 5 feet constructed of 3/4 inch exterior plywood bolted to 4 by 4 inches treated wood posts.

3.	Sign shall be prepared by professional sign painter using either painted exhibit lettering or die cut adhesive applied letters.

4.	Design, style and proportional sizes of lettering, color, and text shall be as shown in Section 01 5001.

5.	Allow no other signs to be displayed without approval of Design Professional or as required by Owner.

*****	Include the following paragraph if school district requires own separate sign.*****

A.	School District Construction Sign.

1.	Furnish project sign and erect on site at location designated by Design Professional.

2.	Construction:  [____________________].

3.	Sign shall be prepared by professional sign painter using either painted exhibit lettering or die cut adhesive applied letters.

4.	Design, style and sizes of lettering, color, and text shall be as shown on [_______] [as provided by Design Professional]..

5.	PSFA sign will be required if there is PSCOC funding in Project.  Allow no other signs to be displayed without approval of Design Professional.

1.18	BULLETIN BOARD

A.	Furnish and maintain bulletin board adjacent to field office.  Display the following throughout construction period:

2.	State wage rates.

3.	Safety requirements.

4.	Official notices and announcements.

1.19	REMOVAL OF UTILITIES, FACILITIES, AND CONTROLS

A.	Remove temporary above grade and buried utilities, equipment, facilities, and excess materials prior to final inspection.

B.	Clean and repair damage caused by installation of temporary facilities.

PART 2 - PRODUCTS

Not used.

PART 3 - EXECUTION

Not used.

END OF SECTION

[(PROJECT SIGN DRAWINGS FOLLOW)]